CBBS -	COOKBOOK Rev. 3.5 11/28/81

This file is a COOKBOOK for the installation of a Computerized

Bulletin Board System (CBBS).  Any comments, errors, omissions,

etc should be referred to the Chicago CBBS, online at (312)

545-8086, or to Ward Christensen at (312) 849-6279, or Randy

Suess, (312) 545-7535.  <<Note new CBBS # and new # for Randy>>

Most of the comments are SUGGESTIONS.  For example, ideas are

presented for the format of the BULLETIN and WELCOME files.

You are of course free to changes these any way you like.

HARDWARE:  THE FOLLOWING ENVIROMNENT IS REQUIRED:

Mainframe: This used to say "S-100" but technically that is not

true; Randy and I are only familiar with S-100.  All the

programming is in 8080 assembler, so the CPU must be 8080 (or

'85) or Z80, etc.  Our original CBBS used a kludge chassis, but

we are now running a Vector 1 and IMSAI CPU.

Memory: 32K.  28 will probably work.  If you have more avail-

able, then you will be able to increase the size of the write

buffer used in message and summary kill (now 2K).  Also, if

your system has many (300 or so) messages and you want to run

the BUILDSUM program to rebuild the summary file, 28K would be

required.  In our 24K system, I ran BUILD with 330 messages,

and only got 4/5 of the way thru the messages before running

out of room.  (We are now a 40K system.) BUILD stores the

summaries in memory, so it could not complete successfully.

Also, if you want to run CBBS under FAST (FX) which is a full

track buffering program, then you will need about 40K.

Modem: CBBS is written for a PMMI, or D. C. Hayes or I.D.S.

modem running thru a DAA.  An external modem can be accomo-

dated, and some generalized code is provided which assumes the

availability of a way to detect ring and carrier, and to go

off-hook.

Disks: Standard size floppy disk and controller capable of

supporting CP/M. Our original system used the Tarbell control-

ler, and an Innovex disk, very successfully).  The software

supports one or two disks.  Smaller disks could be used, but

the number of messages would have to be kept fairly low.

Additional hardware: Local console and interface - This may

consist of a terminal such as a Soroc or ADM-3, and serial

port to access it, or a keyboard-VDM or other similar combina-

tion.  (Note the CP/M BIOS does not have room for a scroll

routine for a memory mapped display such as the VTI or VDM.  We

use a scroll routine burned into a PROM, calling it from the

BIOS).

Software: The system requires the use of the CP/M operating

system.  (or, technically, a "compatible" system).

The system is running under both CP/M 1.3, 1.4, and 2.0.

I doubt there is any interest in 1.3 any more; The STAT command

in CBBSOPER has been rewritten to support 1.4 and 2.2. Steve

Vinokuroff found that the "extend" routine which positions to

the last sector of a file, backs up 1, then scans for EOF, had

to be changed to back up two sectors (testing for backing to a

"negative" number).

Now, you must decide the mode of operation of the system:

The preferred mode, and the only one I know people have tried,

is to put the modem's I/O directly in CP/M's BIOS.  This is

preferred because it allows remote maintenance of the system

via the XCPM command (eXit to CPM).  It requires that both the

modem, and the local console I/O be included in the 3 routines:

console status, console in, and console out.  The original

system made use of a sense switch (80H bit value on port 3) to

determine if the system was in remote (modem I/O) mode, and if

not, dealt only with the local console.  Note that a system

with no

front panelcan be accomodated as the original system did, by

putting a switch on an unused parallel port, such as on a 3P+S,

as we did.  If this mode is chosen, do the following:

Modify your BIOS console status, in, and out, to look like the

ones in CEBIOS.ASM on the supplied disk. This BIOS has the

following required features:

1.   Console status, console in, and console out all deal with

the local console, and if the sense switch is on, with the

modem.

2.   Console status is reflected as true if loss of carrier

occurs.  This prevents hanging in the status routine, after

loss of carrier.

3.   Character in sends back an 0FFH if loss of carrier occurs.

CBBS.ASM checks for this.  This implies you "ANI 7FH" for all

normal ASCII characters, to ensure an FF won't be seen except

due to carrier loss.

----------------

If in addition, you want the system to automatically come up

running the CBBS program from cold boot:

4.   Cold boot checks the sense switch, and if in remote mode,

does not go into CP/M command mode, but instead loads CBBS.COM

and transfers control to it.  Thus the user never sees or is

allowed access to the CP/M environment.

Note that with CP/M 1.4 and higher, the auto-load feature

whereby you patch the CBBS command into CCP's input buffer (at

CCP+7: 7 is the length <e.g. 4 for "CBBS"> and 8-on is the

upper case, actual command to be executed) is much "cleaner"

than our original "load it yourself" trick. With 1.4 and auto-

boot, when you JMP to CCP, the command is automatically exe-

cuted.  In warm boot, JMP to CCP+3 which skips the auto-boot

into CP/M.  This is so the "XCPM" command would allow you to

get into CP/M in operator mode.  We supply a Tarbell single

density BIOS for CP/M 1.4.

An alternate mode of operation is to put the modem I/O

directly  in the CBBS.ASM program.  I don't know of anyone who

has done this.  If you do, the XCPM command should be

nullified.  If XCPM were to be used, the system would go into

CP/M, but would then sit there communicating only with the

local console, and the phone would be off hook. Note that

built-in I/O drivers are not in CBBS.ASM.  The supplied

version is intended to work with modem I/O via CP/M BIOS as

described above.

An "apparently" attractive alternative is to run CBBS under

Dave Jaffe's BYE program, a general-purpose program for remote

access into a CP/M system.  It is distributed on CP/M Users

Group volumes 40 and 46, and via various SIG/M disks, and

remote CP/M systems.  H O W E V E R, CBBS is written to have a

"DISK BUSY" flag, which is tested.  For example, if someone

leaves comments, then hangs up without ending, the disk busy

flag causes a C/R to be written into the file.  With BYE, the

loss of carrier causes BYE to "yank" control away.  This could

leave an un-terminated file open on disk, since message entry,

log, kill, and comments, all "extend" disk files, i.e. over

write the previous EOF (^Z)) characters in the file.

BYE MAY be used, but you may occasionally experience

strangeness in your files.  I don't know a good solution,

except having BYE handle the disk busy switch, returning 0FFH

upon carrier loss, like CBBS expects.  But then when BYE is

running something OTHER than CBBS, it should handle the

carrier loss itself.  An as yet unsolved problem.

------------------

Obtain hard copy listings of all files, particularly the

CBBS.ASM source files: CBBSxxxx.ASM.

Study these listings until you are familiar with the basic

operation of the program.

Select the options, such as if you have a log teletype, and

make the appropriate source changes (TTY EQU TRUE or TTY EQU

FALSE).  Also change the equates for the TTY, which are now:

status port 0, data port 1, ready bit 80H.  The routine which

checks if the TTY is ready does a RZ if it is not ready.  If

your TTY is LOW ready (ours was HI ready) then change this to

RNZ.

If you will be running a system which is able to cold boot

itself, set REINIT to FALSE.  If you just want to type CBBS

and  then have it sit in memory, use REINIT EQU TRUE - when

one  call completes, CBBS just monitors ring detect and

handles the  next call.  It reinitializes itself (clears

operator mode, #  of nulls, etc)

Make other appropriate source changes.  For example, at label

TELLUS is a message which tells the user that a system error

has occurred (such as if the file NEXT isn't found on disk, or

if the disk is full).  This label has associated with it a

message explaining who to call to notify that the system had a

problem.  Change this as appropriate.  Note the availability

of  the FIND command to find any label in the many CBBS .ASM

files:  FIND CBBS*.ASM TELLUS for example.

I eventually hope to pull ALL system-dependent pieces of CBBS

into a single module, so "technically" you would only have to

change a single .ASM file.  Sorry for not having done that

yet.

Make a running copy of CP/M for the system, having made the

appropriate changes to the BIOS.  It should be at least a 28K

system. (Note this is actually 27K in a Tarbell controlled

system).  You might want to try running CBBS under another

CP/M  system, i.e. without the modem I/O.  CBBSTEST.COM on the

distributed diskettes is just for this purpose.  It has the

SSW input NOPed, as well as modem I/O, etc.

Assemble CBBS and place the COM files on the disk which will

run in the CBBS system itself.

Pip over the files required by CBBS:

Location in a

NAME	  2 drive system:

BULLETIN	B:

WELCOME		B:

NEXT		A:

FIRSTIME	B:

ENTINTRO	B:

ENTRHELP	B:

FUNCTION	A:

HELP		B:

HELP2		B:

HELP.IDX	B:

PASSWORD	B:

SCANHELP	B:

Initialize your PASSWORD file to contain the operator access

password: any word or phrase terminated by C/R.  CBBSPASS is

used in the test system supplied.

Initialize file NEXT to contain:

00020(cr)(lf)   <-- next message number (MUST have leading 0s)

1(cr)(lf)   <-- next caller number

10(cr)(lf)   <-- number of active messages

300(cr)(lf)   <-- Maximum number of active msgs

(ctl-z)

00020 is the starting message number;  This is assuming that

you will leave the first 20 message numbers open for permanent

CBBS operational messages.  See the sample CBBS supplied for

the contents of these messages.  You might even wish to just

edit these messages, deleting the other test messages, and do

a  BUILD to get the summary, and use this for the start of

your  system.

The second line is the next caller number, treated as a lead-

ing zero suppressed ASCII 5 digit number, which is incremented

and printed each time someone calls the system. It is also

written to the LOG before the person's name.

The third line is the number of active messages.  This may be

manually initialized if you have a small number of messages,

or  you may execute "T SUMMARY #" which prints the number of

lines  in the summay file, then divide that number printed by

2, to  get the number of active msgs.  The summary may,

however, have flag-deleted messages, so this number won't be

absolutely correct.  The best way is to run BUILDSUM which

will not only build a new summary file, but tell you exactly

how many messages there are.

Back to file NEXT: The fourth line is a maximum number of

active messages. This prevents the disk over-filling so that

kill wouldn't work.  NOTE this is compared character by

character with the line above it, so both must be leading zero

suppressed numbers.

Edit the file WELCOME to put in your own welcome message.

Leave the comments about the control characters.  Please leave

the (R) next to "CBBS(R)" and "Computerized Bulletin Board

System"(R) as we have trademarked the terms.

Edit the BULLETIN file.  Keep the top line up to date as to

how  many lines it contains, and when it was last updated.

You'll  have to decide yourself how big you want to let it

grow, and  what it will contain.  Rather than having a super

big bulletin,  its better to refer people to specific messages

by number, in  the bulletin.

Note: With CBBS 3.5, there is now a NEWS file, which may be

considered to be an extension of the BULLETIN file.  You may

wish to make reference to it in the BULLETIN, e.g. for more

news about the system, type: NEWS

Edit the file FIRSTIME to make it appropriate to your environ-

ment.  Feel free to add any questions which you might like to.

A question starts on a line by itself, starting with the char-

acter [.  The question may not be more than 1 line long, and

is  terminated by cr/lf, without any '?'. The information to

be  typed in response to a Y answer to the question is entered

directly following the question line.  When you have completed

the answer, type a ] character which signals the end of the

question.  This is so if the caller replies N to the question,

the CBBS software will skip past the ] character.  Typing the

reply ends with the ']' character, so it should be on a line

by  itself, not on the end of the previous line.

You will want to edit the first 20 messages, to put in infor-

mation about your system.  For example message 1 contains the

system history.  Other than acknowledging "Ward and Randy" as

the original source, make it whatever you feel is appropriate.

Also, update the line count in the header.  Then remember to

run BUILDSUM after changing these messages, so the SUMMARY

matches your messages.

Now try the system from the local console or modem, which ever

you feel more comfortable with.  Try all commands, and all

options (first time user, help file, full/half duplex, bulle-

tin, welcome, etc.) to make sure all the functions work and

all  the files are present.  Even though you may be familiar

with CBBS operation, it is probably best to STAY OUT OF EXPERT

MODE TO be sure all the prompts are OK.

Now try entering a message.  Use all of the enter functions,

such as retyping a line, editing a line, insert, delete, etc.

Abort out and make sure that works.  Then leave a legitimate

message.

Your system should now be ready to go 'on the air'.

DAY TO DAY OPERATIONS

If there are multiple operators on the system, they will prob-

ably want to leave messages for each other, which they would

like to be more "private" than just leaving a regular message

permits.  The most obvious way is to leave a comment, since

only the operators can view them via the TYPE LOG command of

operator mode.  This is inconvenient, however, as it becomes

hard to find the other operator's comments among the user's

comments. For this reason, a special function "NOTE" is imple-

mented when in operator mode.  Typing NOTE results in being

able to key lines into a file NOTES.  It works just like the

comments function, i.e. enter a null line (two returns in a

row) to terminate input. 'FROM firstname:' is placed at the

front of the notes.  Repeated use of NOTE results in

additional  notes appended to the original.  Each is date/time

stamped.  To avoid time/date stamping, use NOTEN.  When using

NOTE, the other operator then only has to issue the command

TYPE NOTES (or TYPE B:NOTES, etc) to see the notes, and the

command ERA NOTES (or ERA B:NOTES, etc) when they have been

seen.  When an operator uses the PASS command to get into

operator mode, the the NOTES file is automatically typed if

there is one on disk.  Control-C will skip a single note,

Control-K will skip them all.  The NOTE function is even

useful when there is only one operator - to leave notes to

yourself!

The operators will want to frequently see the LOG files.  Get

into operator mode (via PASS command, and the correct

password).  Use the TYPE command to TYPE LOG. If the LOG file

is  large, control-x may be used to skip blocks of 10 lines,

or  Control-C to skip to the next comment.  Each comment,

unsuccessful message kill, password violation, or request for

a keyword not in the HELP file, has a "]" character before it.

This is the character which control-c skips to.  The name of

the person leaving the comment is logged again at the end of

the comment.  CBBS didn't used to do this, so you didn't know

who the comment was from.  If you have a clock, and want to

see comments after a certain date, use "type b:log,mm/dd"

since any string following TYPE will be skipped to.  ^C skips

to the next occurrence.  I use "TYPE c:log,ward christensen"

for example, so that I can skip to right AFTER MY OWN last

LOGIN.  I let 2 lines print, to see what date, and if I see

its not the most recent previous login, press  again to skip

to the next one.

When the LOG file nears 1 extent (DIR B:LOG shows over 100 or

so sectors) we suggest renaming the LOG file.  We use Lyymmdd

as the new name.  This is to minimize the time it takes to

access the file online.  (Note that in CBBS 3.4 or so, a more

efficient algorithm was introduced, so large files are not as

bad as they used to be.  EXTEND looks for the highest extent,

rather than scanning thru each extent after the first.  Also,

this will be an operator convenience in typing the LOG file as

it won't be as long.  A new LOG or KILLED file is

automatically created if none is found.

Note the TYPE command allows a character string search.  I

have  a notebook in which I type the current caller number

every time  I call in.  Then, the next time I call in, I can

just TYPE  B:LOG,nnnnn where nnnnnn is the caller number the

last time I  called in.  See more comments on limitations of

TYPE in  CBBSOPER.DOC.

Periodcally pull the LOG files and the KILLED files off to

another disk, as the online disk gets full.

If it becomes necessary to edit a message file, such that the

message header would change, be sure to edit the SUMMARY file

to reflect the changes.   An alternative would be to run

BUILDSUM to rebuild the summary, but make sure you have enough

memory to do this.  (The summaries are read into memory by

scanning all message files) (An alternative is to take the

message disk to a larger CP/M system and do the BUILD there).

On CBBS/Chicago, we used the first 20 messages for system use.

This was not done initially, but later the editor was used.

We  suggest you keep this policy. If you have a need such as

this,  or want to create a message without CBBS (i.e. any

message not  at the end) you should know the following about

the format of  each message header:

MESSAGE	HEADER

A bell chaaracter (07H).

Then the 5 digit message number

a comma

the date (m or mm, '/', d or dd, '/', yy)

a comma

who the message is from (SYSTEM OPERATORS, etc)

a comma

Who the message is to

new line (i.e. crlf)

subject

comma

password (the word NONE if you don't want it killable)

Together these would look like this: (where ^G is a control-G)

NOTE the header MUST be in UPPER CASE so that the S and Q

string search commands work. (S;1,t=ward looks for WARD in

upper case only)

^G00003,05/08/78,SYSTEM OPERATORS,ALL

SAMPLE MESSAGE,NONE

When the disk or directory becomes fairly full, you will have

to kill off messages.  We keep messages for 2 months on

CBBS/Chicago.  More lately, we have decided to keep 320 active

messages.  This means keeping them about 50 days. Use your own

judgement.

N-O-T-E

The message kill function works like an editor:  a file is

created, and the message file is passed thru memory, to the

temporary file, with the requested message deleted.  This

means you must have enough room on disk for this temporary

file, i.e. enough room for a second summary file (since the

summary will typically be bigger than the largest message

file, once you get a few messages).  REV: CBBS 3.4 no longer

passes the summary file thru memory on every Kill.  The PURGE

command must be used to physically delete flagged summary

messages, or alternatively, run BUILDSUM to "blow away" the

summary file and rebuild it (IF you have enough memory to hold

the entire summary along with CP/M and BUILDSUM.COM).

NOTE that this version of CBBS no longer packs messages. Mes-

sage numbers will never be reused, except the low numbered

messages which are "reused" by editing them.  When

CBBS/Chicago reached 10,000 messages, a few people asked that

we start over at 100 or so, but we "like" seeing the 10000+,

numbers, and with "+" and "-" retrieval, and flagged

retrieval, they are not "that" inconvenient.

Well, that's it!! Hope this helps get you on the air, and have

fun!!  Don't forget to back up the disk periodically "just in

case".  We do it once a week + whenever software changes are

implemented.

As stated in the Terms and Conditions, you're on your own, but

we wouldn't refuse an occasional chat (question, etc) to:

Ward Christensen (312) 849-6279

Randy Suess (312) 545-7535

NOTE that CBBS is licensed to you.  I was recently

disappointed  to hear the entire CBBS source files are in a

local "scroung- er's" "hot" program library.

..and did we remember to say "THANKS" for supporting CBBS???

Ward C. 11/25/81